OX-ZT412G

4G Helmet Camera

- Features

- High Level Protection

- -IP66, adaptive to various environments

- -2 meters drop protection

- Excellent Video Transmission

- -4G / Wi-Fi transmission, improves the video stream continuity under unstable network conditions

- -H.264 / H.265 video codec, saves up to 50% bandwidth and storage

- High Quality Image

- - Up to 2K HD video quality, 2560 x 1440p @ 30fps

- - Higher resolution camera, supports up to 64M

- Professional lighting features

- -Flashlight flood mode reaches 125lm

- -Flashlight spot mode reaches 80lm

- Abundant Functions and Features

- -2.0” IPS touchscreen display

- -Gesture lighting control

- -Dual-camera design with fixed focus and zoom modes

- -Starlight night vision main camera for clearer nighttime video

- -140 degree wide-angle camera

- Multimedia function:

- -Support cluster PTT intercom

- -Support functions such as video recording, QR code scanning, TTS, etc

- -Support file upload to cloud servers and support breakpoint resume function

- -Support AI algorithms such as facial recognition, license plate recognition, etc

Specifications

| Model | OX-ZT412G |  |
| --- | --- | --- |
| Operation System | Operation System | Android 12.0 |
| CPU | CPU | MT6769 8 Cores CPU 64 bit, 2.0GHz. |
| Memory | RAM+ROM | 3GB+32GB by default(optional 4GB+64GB/4GB+128GB/4GB+256GB) |
| Main Lens (1st) | Image Sensor | 1/3" progressive scan CMOS |
|  | Max. Recording Resolution | 2560 × 1440 |
|  | FOV | ≧ 140° |
|  | Focal Length | 1.9 mm fixed lens |
|  | Aperture | 2.3±5% |
| 2nd Lens | 2nd Lens | 8 million AF |
| Video | Video Compression | H.264, H.265 |
|  | Main Stream | 2560×1440/30fps, 1920×1080/30fps, 1280×720/30fps, 640×480/30fps |
|  | Sub-Stream | 2560×1440/30fps, 1920×1080/30fps, 1280×720/30fps, 640×480/30fps |
|  | Video Format | MP4, PS |
|  | Pre-record & Post-record | Pre-record: 60/10/5/3s,(Optional) |
|  |  | Post-record: 60/10/5/3s(Optional) |
| Image | Resolution | 64M, 48M, 32M, 16M, 8M, 4M |
|  | Image Format | JPEG |
| Audio | Audio Compression for Video Recording | AAC, G711 |
|  | Audio Compression for Audio Recording | MP3,AMR,WAE |
|  | Audio Compression for Group Intercom | OPUS,G711 |
|  | Audio Bit Rate | AMR (12 kbps), AAC (128 kbps), MP3 (128 kbps), G711 (192 kbps),OPUS(13 kbps) |
| Screen | Screen Size | 2.0" |
|  | Touchscreen | Supported |
|  | Display Resolution | 640 × 480 |
|  | Screen Brightness | Typical value: 350 cd/m² |
| Lighting function | Professional lighting | Flashlight flood mode reaches 125lm,Flashlight spot mode reaches 80lm |
|  | Laser Lighting | Supports laser positioning |
|  | Gesture lighting | Supported |
| Network Parameters | Wireless Network | 2.4G/5.8G Wifi 5 ac, support IEEE802.11 ac/a/b/g/n |
|  | Bluetooth | BT5.0 |
|  | Positioning Mode | GPS |
|  | 4G | LTE FDD: B1/B2/B3/B4/B5/B7/B8/B20/B28A |
|  |  | LTE TDD: B34/B38/B39/B40/B41 |
|  |  | WCDMA：B1/B5/B8 |
| Others Parameters | Gyroscope | Built in |
|  | G-Sensor | Built in |
| Network Protocol | Protocol | RTSP, Onvif, MCS8, FTP |
|  | API | SDK |
| Security | Encryption | AES256 |
|  | Watermark | Supported |
| Software Functions | Smart Functions | Face Picture Capture(Optional) |
|  |  | Face Picture Recognition(Optional) |
|  |  | License Plate Recognition(Optional) |
|  | Basic Event | SOS alarm |
|  | Group Intercom | Supported |
|  | Voice Broadcast | Supports TTS voice broadcast(Optional) |
|  | File Upload | Supports OSS object storage and file upload |
|  | Video Intercom | Supports video calls between multiple devices(Optional) |
|  | Voice Call | Supports voice calls between multiple devices(Optional) |
|  | Message Delivery | Supports platform-to-device messaging of messages, files, and other content(Optional) |
|  | Software Upgrade | Supports OTA firmware or app upgrades |
| Battery | Battery Life | Up to 10 hours in 1080p resolution¹ |
|  | Charging Way | Type C |
|  | Capacity | 4000mAh |
|  | Battery Type | Lithium-ion battery |
|  | Charging Time | ≤ 4 h |
| General | Power | 5 V DC, 2.0 A |
|  | Charging Interface | Type-C, Pogo Pin |
|  | Mic | 2 built-in mic(s) |
|  | Speaker | 1 speaker(s),max. power consumption: 1.5 W |
|  | USB | Type-C |
|  | SIM Card | 1 Nano-SIM card(s) |
|  | TF Card | 1 TF card(s),64 /128/256 /512GB optional |
|  | Button | LED Light, Power on/off, Video recording |
|  | Dimensions | 69.3 x 55 x 28mm |
|  | Weight | 220g |
|  | Operating Temperature | -20℃～+55℃ |
|  | Operating humidity | 40%-90% |
|  | Protection Level | IP66 |
|  | Anti-drop level | 2M |
| Warranty | Warranty | 1 year |
| - This is a basic estimation of battery working life only when recording is on. Additional power consumption should be considered when IR, GPS, and/or network connection is on - All battery claims depend on network configuration and many other factors, actual results will vary. - Product parameters may vary, please refer to the actual product. |  |  |